HydroSense

Smart Hydroponic Monitoring System

React Native App — Development Guide by Phase

DOCUMENT OVERVIEW

This document provides a complete, phase-by-phase guide to building the HydroSense plant monitoring app. It covers front-end development, hardware integration, backend setup, AI/ML implementation, and full system integration. Follow each phase in order for the smoothest development experience.

Phase Overview

| Phase | Title | Focus | Output |
| --- | --- | --- | --- |
| 1 | Frontend | React Native UI screens with mock data | Working app skeleton |
| 2 | Hardware | ESP32 firmware + sensor wiring | Live sensor readings |
| 3 | Backend | Node.js API + Firebase database | REST API + real-time DB |
| 4 | AI / ML | TensorFlow Lite plant classifier | Plant stage classification |
| 5 | Integration | Connect all components end-to-end | Fully working system |

PHASE 1    Frontend — React Native UI

Set up the React Native/Expo project and build all five screens with mock (static) data before connecting any live hardware or backend. This lets you verify the entire UI is correct first.

### 1.1  Project Initialization

npx create-expo-app HydroSense

cd HydroSense

npm install @react-navigation/native @react-navigation/bottom-tabs

npm install react-native-screens react-native-safe-area-context

npm install victory-native react-native-svg

npm install axios @react-native-async-storage/async-storage

npx expo install expo-camera expo-image-picker expo-notifications

npx expo start

### 1.2  Project File Structure

Replace the default App.js with the provided code and organize your src/ folder:

- App.js — Root entry point, wraps NavigationContainer

- src/navigation/AppNavigator.js — Bottom tab navigator (5 tabs)

- src/screens/DashboardScreen.js — Main overview screen

- src/screens/MonitoringScreen.js — Sensor readings + charts

- src/screens/CameraScreen.js — Camera capture + growth AI

- src/screens/LogsScreen.js — Historical data table

- src/screens/HealthScreen.js — System hardware status

- src/components/SharedComponents.js — Reusable Card, Badge, etc.

- src/utils/theme.js — Colors, spacing, sensor ranges

- src/utils/api.js — API functions (mock data in Phase 1)

### 1.3  Screen Checklist

Build each screen and verify it renders with mock data:

- Dashboard: sensor cards (2x2), growth stage bar, individual temp sensors, alert banners

- Monitoring: sparkline charts for temp/humidity/pH/TDS, color indicators, time range tabs

- Camera Analysis: image preview placeholder, growth stage boxes, nutrient bars + modal

- Records & Logs: filterable FlatList table with timestamps and status badges

- System Health: ESP32 status, SD card bar, sensor array list with green/red dots

### 1.4  Key Dependencies

All packages needed and their purpose:

| Package | Purpose |
| --- | --- |
| @react-navigation/bottom-tabs | 5-tab bottom navigation bar |
| react-native-safe-area-context | Handles notch/home indicator padding |
| victory-native / react-native-svg | Sensor time-series charts |
| expo-camera / expo-image-picker | Plant photo capture (Phase 4) |
| expo-notifications | Push alert when sensor out of range |
| axios | HTTP calls to backend (Phase 5) |
| @react-native-async-storage | Cache last readings offline |

PHASE 2    Hardware — ESP32 Firmware

Wire all sensors to the ESP32 and write firmware that reads sensor data every 60 seconds and sends it to the backend API via WiFi.

### 2.1  Hardware Components

- ESP32-S3 development board

- 3x DHT11 sensors — temperature & humidity

- Analog pH sensor + signal conditioning module

- TDS (EC) sensor module

- OV2640 Camera Module (for ESP32-CAM variant)

- MicroSD card module + SD card

- Jumper wires, breadboard, 3.3V power rail

### 2.2  Wiring Guide

| Sensor | ESP32 Pin | Notes |
| --- | --- | --- |
| DHT11 #1 (Data) | GPIO 4 | 10kΩ pull-up to 3.3V |
| DHT11 #2 (Data) | GPIO 5 | 10kΩ pull-up to 3.3V |
| DHT11 #3 (Data) | GPIO 18 | 10kΩ pull-up to 3.3V |
| pH Sensor (Analog) | GPIO 34 (ADC1_CH6) | Use voltage divider if >3.3V |
| TDS Sensor (Analog) | GPIO 35 (ADC1_CH7) | Use voltage divider if >3.3V |
| SD Card (SPI MOSI) | GPIO 23 | SPI bus |
| SD Card (SPI MISO) | GPIO 19 | SPI bus |
| SD Card (SPI CLK) | GPIO 14 | SPI bus |
| SD Card (CS) | GPIO 15 | Chip select |

### 2.3  Arduino Firmware Steps

- Install Arduino IDE and add ESP32 board: https://dl.espressif.com/dl/package_esp32_index.json

- Install libraries: DHT sensor library, ArduinoJson, WiFi, HTTPClient, SD

- Write loop() that reads all 3 DHT11s, averages temperature, reads pH/TDS via ADC

- Calibrate pH using 4.0 and 7.0 buffer solutions; calibrate TDS with known solution

- Build JSON payload: { "temp": {...}, "humidity": 62.1, "ph": 6.29, "tds": 925 }

- POST payload to http://<your-backend>/api/readings every 60 seconds

- Also write same payload to SD card as CSV backup (date, values)

- Serial.println() all values to monitor before going live

PHASE 3    Backend — Node.js + Firebase

Build the REST API server that receives sensor data from the ESP32 and serves historical data to the React Native app. Firebase Firestore provides real-time sync.

### 3.1  Project Setup

mkdir hydrosense-backend && cd hydrosense-backend

npm init -y

npm install express cors dotenv firebase-admin

npm install -D nodemon

### 3.2  API Endpoints

| Method | Endpoint | Description | Caller |
| --- | --- | --- | --- |
| POST | /api/readings | Receive sensor data from ESP32 | ESP32 |
| GET | /api/readings/latest | Return most recent reading | Mobile App |
| GET | /api/readings?limit=N | Return last N readings for logs | Mobile App |
| GET | /api/readings/chart?var=temp | Return chart data array | Mobile App |
| POST | /api/classify | Accept base64 image, return stage | Mobile App |
| POST | /api/nutrients | Save user nutrient input | Mobile App |
| GET | /api/system/health | Return system status object | Mobile App |

### 3.3  Firebase Setup Steps

- Go to console.firebase.google.com → Create project "hydrosense"

- Enable Firestore Database (start in test mode for development)

- Create collection "readings" — each doc: { temp, humidity, ph, tds, timestamp }

- Go to Project Settings → Service Accounts → Generate new private key

- Save as serviceAccount.json in backend project root

- Initialize firebase-admin with service account in your server.js

### 3.4  Deployment

- Recommended free hosting: Railway.app or Render.com

- Set environment variables: FIREBASE_PROJECT_ID, PORT

- Copy the deployed URL into src/utils/api.js as API_BASE

PHASE 4    AI / ML — Plant Classification

Implement the growth stage classifier that takes a plant photo and returns: Seedling, Vegetative, or Fruiting, with a confidence percentage.

### 4.1  Option A — On-Device (TensorFlow Lite)

Best for: offline use, faster results, no server cost.

- Collect 100+ images per class: Seedling, Vegetative, Fruiting

- Label using LabelImg or Roboflow and split into train/val/test sets

- Fine-tune MobileNetV2 with transfer learning in Python/TensorFlow

- Convert: tflite_convert --output_file=model.tflite --saved_model_dir=./saved_model

- Copy model.tflite to React Native project assets/ folder

- Install: npm install react-native-tflite

- Call model in CameraScreen after image capture, display results

### 4.2  Option B — Python Flask API (Easier to Start)

Best for: faster setup, easier debugging during development.

- pip install flask tensorflow pillow flask-cors

- Create app.py with POST /classify endpoint that accepts base64 image

- Load your trained .h5 model and preprocess the incoming image

- Return JSON: { stage: "Vegetative", confidence: { Seedling: 8, Vegetative: 90, Fruiting: 24 } }

- Deploy to Hugging Face Spaces (free) or any Python host

- Call from React Native via Axios in getGrowthClassification() in api.js

### 4.3  Camera Integration in React Native

import * as ImagePicker from 'expo-image-picker';

const result = await ImagePicker.launchCameraAsync({

base64: true,

quality: 0.7,

allowsEditing: true,

aspect: [4, 3],

});

if (!result.cancelled) {

const classification = await getGrowthClassification(result.base64);

setGrowth(classification);

}

PHASE 5    Integration — Connect Everything

Replace all mock data with real live data. Wire the React Native app to the backend, connect to Firebase for real-time updates, and enable push notifications.

### 5.1  Replace Mock Data with Real API Calls

In src/utils/api.js, uncomment the real fetch calls and delete the mock returns:

// BEFORE (Phase 1):

// return MOCK_READINGS;

// AFTER (Phase 5):

const res = await api.get('/readings/latest');

return res.data;

### 5.2  Firebase Real-Time Listener

- Install Firebase JS SDK: npm install firebase

- Initialize Firebase in a new src/utils/firebase.js file

- In DashboardScreen.js, subscribe to onSnapshot() for live updates

- Unsubscribe on component unmount to avoid memory leaks

### 5.3  Push Notifications

- Register device token on app start using Expo Notifications

- Store token in backend/Firebase per device

- In backend: when a POST /readings comes in, check values against optimal ranges

- If out of range, send push via Expo Push API to all registered tokens

- Test by setting a very tight range temporarily

### 5.4  End-to-End Test Checklist

- ESP32 powers on and connects to WiFi → serial monitor shows "WiFi connected"

- ESP32 POSTs to backend → Firestore shows new document in readings collection

- App Dashboard auto-updates sensor values within 60 seconds

- Capture plant photo → classification returns a result with confidence

- Change a sensor value outside optimal range → push notification appears on phone

- Logs screen shows last 50 entries with correct timestamps

- System Health shows all 5 sensors as Operational

- Disconnecting ESP32 → System Health shows Offline within 2 minutes

### 5.5  Build Release APK

- Install EAS CLI: npm install -g eas-cli

- Login to Expo account: eas login

- Configure build: eas build:configure

- Build Android APK: eas build --platform android --profile preview

- Download APK from Expo dashboard and install on your device

- For iOS: eas build --platform ios (requires Apple Developer account)

## Appendix — Sensor Calibration Notes

### pH Sensor Calibration

- Prepare pH 4.0 and pH 7.0 buffer solutions

- Submerge probe in pH 7.0 solution, record ADC voltage (V_neutral)

- Submerge probe in pH 4.0 solution, record ADC voltage (V_acid)

- Slope = (7.0 - 4.0) / (V_neutral - V_acid)

- pH = slope * (V_measured - V_neutral) + 7.0

### TDS Sensor Calibration

- Use a known TDS reference solution (e.g., 442 ppm NaCl standard)

- Measure ADC voltage in solution, note the reading

- Apply correction factor: TDS_actual = TDS_raw * (reference_ppm / measured_ppm)

- Temperature-compensate: TDS_corrected = TDS_measured / (1 + 0.02 * (T - 25))

### DHT11 Notes

- DHT11 has ±2°C accuracy — average 3 sensors for better reliability

- Sample no faster than once per 2 seconds (DHT11 minimum interval)

- Replace with DHT22 for ±0.5°C accuracy if budget allows

## Appendix — Optimal Ranges Reference

| Parameter | Seedling | Vegetative | Fruiting | Unit |
| --- | --- | --- | --- | --- |
| Temperature | 18–24 | 20–28 | 22–30 | °C |
| Humidity | 60–70 | 55–75 | 50–65 | % |
| pH Level | 5.5–6.5 | 5.8–7.0 | 6.0–6.8 | pH |
| TDS Nutrients | 200–400 | 800–1200 | 1000–1500 | ppm |
| Nitrogen (N) | 100 | 200 | 150 | ppm target |
| Phosphorus (P) | 50 | 60 | 100 | ppm target |
| Potassium (K) | 150 | 220 | 300 | ppm target |